SUPPLEMENTARY METHODS AND QUALITY CONTROL

Supporting evidence for the revised Indus Basin crop water-accounting manuscript

# S1. Analytical scope and reproducibility

The analytical support is cropland within the Pakistan portion of the Indus drainage basin. It is not an official canal-command delineation. The crop map describes the 2023 seasonal extent: crop-hectares can exceed unique physical area under double cropping, maize combines spring and autumn cycles, and sugarcane is treated as annual. Water accounting is crop-season based and residual blue ET is not measured irrigation withdrawal.

# S2. Predictor and classification details

The Rabi stack contains 42 predictors and the Kharif stack 47. Predictor families include period-specific Sentinel-2 NDVI, EVI, NDWI, LSWI, and SAVI; full-season Sentinel-2 and Landsat optical bands and indices; Sentinel-1 median VV, VH, and VV-VH; and SRTM elevation and slope. Missing predictors were filled with -0.25 before sampling. Both models used 300 trees, minimum leaf population 2, bag fraction 0.70, default variables per split, no maximum-node limit, and classifier seed 20230709. Reference points were assigned by whole 0.05-degree blocks using a deterministic SHA-256 rule with seed 20230709.

## Table S1. Class-level validation diagnostics

| Season | Class Code | Crop Name | Validation Support | Predicted Support | Correct | Producer Accuracy | User Accuracy | F1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rabi | 1 | Wheat | 467.000 | 549.000 | 438.000 | 0.938 | 0.798 | 0.862 |
| Rabi | 4 | Sugarcane | 125.000 | 115.000 | 97.000 | 0.776 | 0.843 | 0.808 |
| Rabi | 5 | Maize | 214.000 | 203.000 | 193.000 | 0.902 | 0.951 | 0.926 |
| Rabi | 6 | Other cropland | 474.000 | 413.000 | 365.000 | 0.770 | 0.884 | 0.823 |
| Kharif | 2 | Rice | 517.000 | 523.000 | 478.000 | 0.925 | 0.914 | 0.919 |
| Kharif | 3 | Cotton | 320.000 | 339.000 | 284.000 | 0.887 | 0.838 | 0.862 |
| Kharif | 4 | Sugarcane | 436.000 | 454.000 | 397.000 | 0.911 | 0.874 | 0.892 |
| Kharif | 5 | Maize | 587.000 | 614.000 | 491.000 | 0.836 | 0.800 | 0.818 |
| Kharif | 6 | Other cropland | 529.000 | 459.000 | 380.000 | 0.718 | 0.828 | 0.769 |

## Table S2. Season-level validation diagnostics

| Season | Overall Accuracy | Kappa | Balanced Accuracy | Macro F1 | Training Samples | Validation Samples | Validation Blocks | Interpretation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rabi | 0.854 | 0.788 | 0.846 | 0.855 | 5535.000 | 1280.000 | NA | spatial-block holdout; not area-probability sampled |
| Kharif | 0.850 | 0.811 | 0.855 | 0.852 | 9763.000 | 2389.000 | NA | spatial-block holdout; not area-probability sampled |

# S3. Crop-area scenario

The official-area-constrained scenario multiplies mapped mean crop water depth and mapped yield composition by the ratio of official harvested area to mapped crop-season area. It is a first-order structural scenario, not a confidence interval. Because area and production are scaled by the same factor, crop-level VWF remains unchanged while water volume and production scale directly.

## Table S3. Mapped and official-area-constrained crop scenarios

| Crop Name | Mapped Area Ha | Official Harvested Area Ha | Mapped Minus Official Pct | Mapped Total Water Km3 | Official Area Scaled Total Water Km3 | Mapped Total Vwf M3 T |
| --- | --- | --- | --- | --- | --- | --- |
| Wheat | 7,930,296 | 8,477,154 | -6.451 | 26.468 | 28.293 | 1066.851 |
| Rice | 4,156,694 | 2,969,151 | 39.996 | 15.284 | 10.918 | 1548.604 |
| Cotton | 1,293,009 | 1,978,787 | -34.656 | 5.662 | 8.665 | 2076.197 |
| Sugarcane | 1,647,041 | 1,284,439 | 28.230 | 12.965 | 10.110 | 125.629 |
| Maize | 2,770,262 | 1,710,539 | 61.953 | 19.635 | 12.124 | 1237.948 |

# S4. Product-chain structural sensitivity

The primary endpoint combines WaPOR v3 AETI with CHIRPS rainfall; the comparison endpoint combines MODIS ET with ERA5-Land rainfall. Endpoints are reported directly. No uniform interpolation, Monte Carlo distribution, or confidence interval is used.

## Table S4. Product-chain endpoints

| Crop Name | Product Chain | Total Water Mm | Green Water Mm | Blue Water Mm | Total Vwf M3 T | Water Productivity Kg M3 |
| --- | --- | --- | --- | --- | --- | --- |
| Wheat | WaPOR-CHIRPS | 333.753 | 86.571 | 247.182 | 1066.851 | 0.937 |
| Wheat | MODIS-ERA5-Land | 133.591 | 86.845 | 46.746 | 427.028 | 2.342 |
| Rice | WaPOR-CHIRPS | 367.707 | 225.058 | 142.649 | 1548.604 | 0.646 |
| Rice | MODIS-ERA5-Land | 98.240 | 97.696 | 0.544 | 413.738 | 2.417 |
| Cotton | WaPOR-CHIRPS | 437.882 | 197.116 | 240.766 | 2076.197 | 0.482 |
| Cotton | MODIS-ERA5-Land | 116.879 | 113.785 | 3.094 | 554.177 | 1.804 |
| Sugarcane | WaPOR-CHIRPS | 787.152 | 321.902 | 465.251 | 125.629 | 7.960 |
| Sugarcane | MODIS-ERA5-Land | 241.894 | 226.641 | 15.253 | 38.606 | 25.903 |
| Maize | WaPOR-CHIRPS | 708.773 | 371.337 | 337.436 | 1237.948 | 0.808 |
| Maize | MODIS-ERA5-Land | 258.035 | 255.168 | 2.867 | 450.686 | 2.219 |

## Table S5. Direct endpoint structural envelope

| Crop Name | Metric | Primary Wapor Chirps | Comparison Modis Era5 Land | Absolute Difference | Comparison Minus Primary Pct |
| --- | --- | --- | --- | --- | --- |
| Wheat | green_water_mm | 86.571 | 86.845 | 0.274 | 0.317 |
| Wheat | blue_water_mm | 247.182 | 46.746 | -200.436 | -81.088 |
| Wheat | total_water_mm | 333.753 | 133.591 | -200.162 | -59.973 |
| Wheat | green_vwf_m3_t | 276.726 | 277.603 | 0.877 | 0.317 |
| Wheat | blue_vwf_m3_t | 790.125 | 149.426 | -640.699 | -81.088 |
| Wheat | total_vwf_m3_t | 1066.851 | 427.028 | -639.823 | -59.973 |
| Wheat | water_productivity_kg_m3 | 0.937 | 2.342 | 1.404 | 149.831 |
| Wheat | blue_dependency_pct | 74.061 | 34.992 | -39.069 | -52.753 |
| Rice | green_water_mm | 225.058 | 97.696 | -127.362 | -56.591 |
| Rice | blue_water_mm | 142.649 | 0.544 | -142.105 | -99.619 |
| Rice | total_water_mm | 367.707 | 98.240 | -269.467 | -73.283 |
| Rice | green_vwf_m3_t | 947.836 | 411.448 | -536.387 | -56.591 |
| Rice | blue_vwf_m3_t | 600.768 | 2.289 | -598.479 | -99.619 |
| Rice | total_vwf_m3_t | 1548.604 | 413.738 | -1134.866 | -73.283 |
| Rice | water_productivity_kg_m3 | 0.646 | 2.417 | 1.771 | 274.296 |
| Rice | blue_dependency_pct | 38.794 | 0.553 | -38.241 | -98.574 |
| Cotton | green_water_mm | 197.116 | 113.785 | -83.331 | -42.275 |
| Cotton | blue_water_mm | 240.766 | 3.094 | -237.672 | -98.715 |
| Cotton | total_water_mm | 437.882 | 116.879 | -321.003 | -73.308 |
| Cotton | green_vwf_m3_t | 934.614 | 539.506 | -395.108 | -42.275 |
| Cotton | blue_vwf_m3_t | 1141.583 | 14.670 | -1126.912 | -98.715 |
| Cotton | total_vwf_m3_t | 2076.197 | 554.177 | -1522.020 | -73.308 |
| Cotton | water_productivity_kg_m3 | 0.482 | 1.804 | 1.323 | 274.645 |
| Cotton | blue_dependency_pct | 54.984 | 2.647 | -52.337 | -95.185 |
| Sugarcane | green_water_mm | 321.902 | 226.641 | -95.260 | -29.593 |
| Sugarcane | blue_water_mm | 465.251 | 15.253 | -449.998 | -96.722 |
| Sugarcane | total_water_mm | 787.152 | 241.894 | -545.259 | -69.270 |
| Sugarcane | green_vwf_m3_t | 51.375 | 36.172 | -15.203 | -29.593 |
| Sugarcane | blue_vwf_m3_t | 74.254 | 2.434 | -71.819 | -96.722 |
| Sugarcane | total_vwf_m3_t | 125.629 | 38.606 | -87.023 | -69.270 |
| Sugarcane | water_productivity_kg_m3 | 7.960 | 25.903 | 17.943 | 225.412 |
| Sugarcane | blue_dependency_pct | 59.106 | 6.305 | -52.800 | -89.332 |
| Maize | green_water_mm | 371.337 | 255.168 | -116.169 | -31.284 |
| Maize | blue_water_mm | 337.436 | 2.867 | -334.569 | -99.150 |
| Maize | total_water_mm | 708.773 | 258.035 | -450.738 | -63.594 |
| Maize | green_vwf_m3_t | 648.580 | 445.678 | -202.902 | -31.284 |
| Maize | blue_vwf_m3_t | 589.367 | 5.007 | -584.360 | -99.150 |
| Maize | total_vwf_m3_t | 1237.948 | 450.686 | -787.262 | -63.594 |
| Maize | water_productivity_kg_m3 | 0.808 | 2.219 | 1.411 | 174.681 |
| Maize | blue_dependency_pct | 47.608 | 1.111 | -46.497 | -97.666 |

# S5. Fixed-extent temporal context

The 2018-2022 baseline and 2023 values use the fixed 2023 crop masks. They describe hydroclimatic/product variation over a short context window, not historical crop-area change or a long-term climatology.

## Table S6. Fixed-2023-extent temporal context

| Crop Name | Metric | Baseline Years | Baseline N | Baseline Mean | Baseline Sd | Value 2023 | Departure | Departure Pct | Standardized Departure | Scope Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wheat | WaPOR AETI | 2018-2022 | 5.000 | 282.907 | 23.375 | 305.578 | 22.670 | 8.013 | 0.970 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Wheat | CHIRPS rainfall | 2018-2022 | 5.000 | 171.499 | 57.682 | 115.911 | -55.589 | -32.413 | -0.964 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Wheat | Primary blue dependency | 2018-2022 | 5.000 | 48.902 | 11.581 | 59.804 | 10.903 | 22.295 | 0.941 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Rice | WaPOR AETI | 2018-2022 | 5.000 | 447.013 | 18.216 | 385.396 | -61.617 | -13.784 | -3.383 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Rice | CHIRPS rainfall | 2018-2022 | 5.000 | 307.427 | 47.088 | 318.187 | 10.760 | 3.500 | 0.229 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Rice | Primary blue dependency | 2018-2022 | 5.000 | 47.872 | 7.600 | 39.174 | -8.698 | -18.169 | -1.144 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Cotton | WaPOR AETI | 2018-2022 | 5.000 | 501.249 | 38.568 | 408.743 | -92.506 | -18.455 | -2.398 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Cotton | CHIRPS rainfall | 2018-2022 | 5.000 | 197.525 | 86.609 | 202.559 | 5.034 | 2.549 | 0.058 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Cotton | Primary blue dependency | 2018-2022 | 5.000 | 62.038 | 9.703 | 51.626 | -10.412 | -16.783 | -1.073 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Sugarcane | WaPOR AETI | 2018-2022 | 5.000 | 876.973 | 34.337 | 798.408 | -78.564 | -8.959 | -2.288 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Sugarcane | CHIRPS rainfall | 2018-2022 | 5.000 | 384.319 | 78.809 | 359.750 | -24.569 | -6.393 | -0.312 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Sugarcane | Primary blue dependency | 2018-2022 | 5.000 | 59.253 | 7.086 | 57.298 | -1.955 | -3.299 | -0.276 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Maize | WaPOR AETI | 2018-2022 | 5.000 | 713.881 | 16.826 | 665.658 | -48.223 | -6.755 | -2.866 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Maize | CHIRPS rainfall | 2018-2022 | 5.000 | 642.683 | 63.916 | 574.696 | -67.986 | -10.579 | -1.064 | fixed validated 2023 crop extent; short product context, not long-term climatology |
| Maize | Primary blue dependency | 2018-2022 | 5.000 | 32.323 | 4.722 | 31.807 | -0.516 | -1.595 | -0.109 | fixed validated 2023 crop extent; short product context, not long-term climatology |

# S6. Hotspot threshold sensitivity

The district score adds one point each for high blue-water volume, high total VWF, and low WP. Threshold pairs 70/30, 75/25, 80/20, and 90/10 were applied both to all 108 computable districts and to the 76 districts with at least 25,000 mapped crop-hectares. These are internal diagnostic priorities, not environmental or legal thresholds.

## Table S7. Hotspot-count sensitivity

| Scope | District Count | Threshold Pair | High Quantile | Low Quantile | Blue Volume Cutoff Km3 | Total Vwf Cutoff M3 T | Water Productivity Cutoff Kg M3 | Score 0 Count | Score 1 Count | Score 2 Count | Score 3 Count | Priority Score Ge 2 Count |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all computable districts | 108 | 70/30 | 0.700 | 0.300 | 0.606 | 1658.319 | 0.603 | 42 | 33 | 33 | 0 | 33 |
| all computable districts | 108 | 75/25 | 0.750 | 0.250 | 0.713 | 2012.244 | 0.497 | 54 | 27 | 27 | 0 | 27 |
| all computable districts | 108 | 80/20 | 0.800 | 0.200 | 0.818 | 2302.369 | 0.434 | 64 | 22 | 22 | 0 | 22 |
| all computable districts | 108 | 90/10 | 0.900 | 0.100 | 1.181 | 3122.410 | 0.320 | 86 | 11 | 11 | 0 | 11 |
| districts with mapped area >=25,000 ha | 76 | 70/30 | 0.700 | 0.300 | 0.791 | 988.038 | 1.015 | 31 | 22 | 22 | 1 | 23 |
| districts with mapped area >=25,000 ha | 76 | 75/25 | 0.750 | 0.250 | 0.996 | 1148.249 | 0.872 | 38 | 19 | 19 | 0 | 19 |
| districts with mapped area >=25,000 ha | 76 | 80/20 | 0.800 | 0.200 | 1.139 | 1444.592 | 0.692 | 44 | 16 | 16 | 0 | 16 |
| districts with mapped area >=25,000 ha | 76 | 90/10 | 0.900 | 0.100 | 1.281 | 2451.910 | 0.408 | 60 | 8 | 8 | 0 | 8 |

## Table S8. District hotspot stability (first 40 rows; complete CSV supplied separately)

| Scope | Gaul1 Name | Gaul2 Name | Mapped Area Ha | Threshold Tests | Priority Count | Minimum Score | Maximum Score | Priority Stability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all computable districts | Khyber Pakhtunkhwa | Buner | 64,589 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Islamabad | Islamabad | 32,749 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Khyber | 15,704 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Orakzai | 14,550 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Abbottabad | 14,007 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | South Waziristan | 11,463 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Karak | 10,357 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Batagram | 6,423 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Shangla | 5,773 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Khyber Pakhtunkhwa | Tor Ghar | 1,762 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Balochistan | Pishin | 154 | 4.000 | 4 | 2.000 | 2.000 | priority under all thresholds |
| all computable districts | Punjab | Rawalpindi | 246,278 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Haripur | 66,371 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Bajaur | 59,996 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Lower Dir | 52,253 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Mansehra | 43,831 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Swat | 43,208 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Kurram | 29,969 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Hangu | 24,869 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Kachhi | 21,531 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Ziarat | 6,593 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Kolai Palas Kohistan | 1,072 | 4.000 | 3 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Jhal Magsi | 40,466 | 4.000 | 2 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Mohmand | 22,926 | 4.000 | 2 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Sibi | 20,280 | 4.000 | 2 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | Upper Dir | 13,051 | 4.000 | 2 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Khyber Pakhtunkhwa | North Waziristan | 13,000 | 4.000 | 2 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Punjab | Chakwal | 188,878 | 4.000 | 1 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Nasirabad | 166,279 | 4.000 | 1 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Killa Saifullah | 24,526 | 4.000 | 1 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Musakhel | 9,878 | 4.000 | 1 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Mastung | 2,891 | 4.000 | 1 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Balochistan | Sherani | 1,090 | 4.000 | 1 | 0.000 | 2.000 | threshold-sensitive priority |
| all computable districts | Punjab | Jhang | 590,741 | 4.000 | 0 | 1.000 | 1.000 | never priority |
| all computable districts | Punjab | Muzaffargarh | 576,725 | 4.000 | 0 | 1.000 | 1.000 | never priority |
| all computable districts | Punjab | Okara | 552,483 | 4.000 | 0 | 1.000 | 1.000 | never priority |
| all computable districts | Punjab | Faisalabad | 531,342 | 4.000 | 0 | 1.000 | 1.000 | never priority |
| all computable districts | Punjab | Sargodha | 518,491 | 4.000 | 0 | 1.000 | 1.000 | never priority |
| all computable districts | Punjab | Sheikhupura | 510,194 | 4.000 | 0 | 0.000 | 1.000 | never priority |
| all computable districts | Punjab | Vehari | 508,577 | 4.000 | 0 | 1.000 | 1.000 | never priority |

# S7. Yield coverage

Production-dependent indicators were calculated only for positive matched district yields. No unsupported yield was imputed. Of 580 district-crop combinations, 379 were computable and 201 lacked a positive matched yield or mapped crop area.

## Table S9. Yield coverage by crop

| Crop Name | District Crop Rows | Rows With Mapped Crop | Rows With Positive Yield | Computable Rows | Mapped Area Ha | Computable Row Pct |
| --- | --- | --- | --- | --- | --- | --- |
| Cotton | 116.000 | 116.000 | 46.000 | 46.000 | 1,293,009 | 39.655 |
| Maize | 116.000 | 115.000 | 81.000 | 80.000 | 2,770,262 | 68.966 |
| Rice | 116.000 | 114.000 | 78.000 | 78.000 | 4,156,694 | 67.241 |
| Sugarcane | 116.000 | 115.000 | 68.000 | 67.000 | 1,647,041 | 57.759 |
| Wheat | 116.000 | 116.000 | 108.000 | 108.000 | 7,930,296 | 93.103 |

## Table S10. Yield coverage by district (first 50 rows; complete CSV supplied separately)

| Gaul1 Name | Gaul2 Name | Crop Rows | Computable Crop Rows | Mapped Area Ha | Any Computable |
| --- | --- | --- | --- | --- | --- |
| Balochistan | Barkhan | 5.000 | 3.000 | 39,240 | 1.000 |
| Balochistan | Dera Bugti | 5.000 | 4.000 | 22,754 | 1.000 |
| Balochistan | Duki | 5.000 | 0.000 | 16,889 | 0.000 |
| Balochistan | Harnai | 5.000 | 3.000 | 3,163 | 1.000 |
| Balochistan | Jaffarabad | 5.000 | 3.000 | 137,821 | 1.000 |
| Balochistan | Jhal Magsi | 5.000 | 2.000 | 40,466 | 1.000 |
| Balochistan | Kachhi | 5.000 | 2.000 | 21,531 | 1.000 |
| Balochistan | Kalat | 5.000 | 1.000 | 3,603 | 1.000 |
| Balochistan | Khuzdar | 5.000 | 3.000 | 32,972 | 1.000 |
| Balochistan | Killa Saifullah | 5.000 | 2.000 | 24,526 | 1.000 |
| Balochistan | Kohlu | 5.000 | 2.000 | 11,602 | 1.000 |
| Balochistan | Lasbela | 5.000 | 2.000 | 16 | 1.000 |
| Balochistan | Lehri | 5.000 | 0.000 | 11,313 | 0.000 |
| Balochistan | Loralai | 5.000 | 3.000 | 14,367 | 1.000 |
| Balochistan | Mastung | 5.000 | 1.000 | 2,891 | 1.000 |
| Balochistan | Musakhel | 5.000 | 3.000 | 9,878 | 1.000 |
| Balochistan | Nasirabad | 5.000 | 3.000 | 166,279 | 1.000 |
| Balochistan | Pishin | 5.000 | 1.000 | 154 | 1.000 |
| Balochistan | Quetta | 5.000 | 1.000 | 27 | 1.000 |
| Balochistan | Shaheed Sikandarabad | 5.000 | 0.000 | 2,560 | 0.000 |
| Balochistan | Sherani | 5.000 | 2.000 | 1,090 | 1.000 |
| Balochistan | Sibi | 5.000 | 3.000 | 20,280 | 1.000 |
| Balochistan | Sohbatpur | 5.000 | 0.000 | 69,023 | 0.000 |
| Balochistan | Zhob | 5.000 | 2.000 | 7,671 | 1.000 |
| Balochistan | Ziarat | 5.000 | 2.000 | 6,593 | 1.000 |
| Islamabad | Islamabad | 5.000 | 1.000 | 32,749 | 1.000 |
| Khyber Pakhtunkhwa | Abbottabad | 5.000 | 2.000 | 14,007 | 1.000 |
| Khyber Pakhtunkhwa | Bajaur | 5.000 | 3.000 | 59,996 | 1.000 |
| Khyber Pakhtunkhwa | Bannu | 5.000 | 4.000 | 47,372 | 1.000 |
| Khyber Pakhtunkhwa | Batagram | 5.000 | 3.000 | 6,423 | 1.000 |
| Khyber Pakhtunkhwa | Buner | 5.000 | 4.000 | 64,589 | 1.000 |
| Khyber Pakhtunkhwa | Charsadda | 5.000 | 4.000 | 73,660 | 1.000 |
| Khyber Pakhtunkhwa | Chitral Lower | 5.000 | 3.000 | 2,783 | 1.000 |
| Khyber Pakhtunkhwa | Chitral Upper | 5.000 | 0.000 | 1,657 | 0.000 |
| Khyber Pakhtunkhwa | D. I. Khan | 5.000 | 5.000 | 244,506 | 1.000 |
| Khyber Pakhtunkhwa | Hangu | 5.000 | 4.000 | 24,869 | 1.000 |
| Khyber Pakhtunkhwa | Haripur | 5.000 | 3.000 | 66,371 | 1.000 |
| Khyber Pakhtunkhwa | Karak | 5.000 | 2.000 | 10,357 | 1.000 |
| Khyber Pakhtunkhwa | Khyber | 5.000 | 3.000 | 15,704 | 1.000 |
| Khyber Pakhtunkhwa | Kohat | 5.000 | 4.000 | 39,270 | 1.000 |
| Khyber Pakhtunkhwa | Kohistan Lower | 5.000 | 0.000 | 460 | 0.000 |
| Khyber Pakhtunkhwa | Kohistan Upper | 5.000 | 0.000 | 1,331 | 0.000 |
| Khyber Pakhtunkhwa | Kolai Palas Kohistan | 5.000 | 3.000 | 1,072 | 1.000 |
| Khyber Pakhtunkhwa | Kurram | 5.000 | 3.000 | 29,969 | 1.000 |
| Khyber Pakhtunkhwa | Lakki Marwat | 5.000 | 4.000 | 38,615 | 1.000 |
| Khyber Pakhtunkhwa | Lower Dir | 5.000 | 3.000 | 52,253 | 1.000 |
| Khyber Pakhtunkhwa | Malakand | 5.000 | 4.000 | 25,220 | 1.000 |
| Khyber Pakhtunkhwa | Mansehra | 5.000 | 3.000 | 43,831 | 1.000 |
| Khyber Pakhtunkhwa | Mardan | 5.000 | 4.000 | 124,589 | 1.000 |
| Khyber Pakhtunkhwa | Mohmand | 5.000 | 2.000 | 22,926 | 1.000 |

# S8. Numerical QC identities

Green volume + residual-blue volume = total crop water volume within floating-point tolerance.

Mapped crop area x matched district yield = mapped production.

Total water volume / mapped production = total VWF.

Mapped production mass / total water volume = water productivity.

Unavailable yield remains unavailable rather than being displayed as zero.

# S9. Software and execution record

| Component | Record |
| --- | --- |
| Operating system | Windows-11-10.0.26200-SP0 |
| Python | 3.12.13 |
| Word automation | Microsoft Word 16.0 COM |
| Geospatial processing | Google Earth Engine JavaScript/Python APIs; versions recorded in project environment files where available |
| Classification/block seed | 20230709 (authoritative archived output provenance) |
| Structural sensitivity seed | Not applicable; direct endpoints only |
| Primary CRS | EPSG:32642; source geographic products retained in their documented CRSs |

# S10. Supplementary figures

Figure S1 is the full 116-district blue-VWF ranking retained outside the main manuscript because journal-scale labels are not readable in a standard figure panel. Figure S2 shows threshold sensitivity for the relative district priority score. High-resolution PNG, TIFF, PDF, and SVG versions are supplied where available.